Chapter 20 is a description of the Arduino.

We’ll return to the library in just a moment—trust me, you’ll want to see what happens next. Exciting things are coming. But first, let’s talk about that blinking microcontroller—because now we know it’s important.

But first we should talk a little about this microcontroller, now that we know it’s important and capable of delivering secret messages through its blinking LED.

That microcontroller I found inside the Galaxy Pizza Delivery lunchbox? Here’s what it looks like (insert sketch). This is a common microcontroller that is sometimes called an “Arduino compatible” microcontroller or an “Arduino clone” because it is based on the open source design of the popular Arduino Uno.

If you want to follow along exactly, you can get the same microcontroller I’m using—plus all the other electronics parts in the story—at gavinholloway.com/electronics.

I recommend the Book 1 Master Kit, which has everything you need, including parts for the robot Gavin builds near the end (yes, there’s a robot. And yes, you might build one too!).

Here’s how you program a microcontroller like mine to make the LED blink a secret message.

Step 1) You’ll need to download the Arduino IDE software to your computer. “IDE” stands for Integrated Development Environment (fancy words for “coding playground”), and it’s how engineers, makers, and other people who change the world program their microcontrollers. You can go to gavinholloway.com/software for instructions on how to download the Arduino IDE.

Step 2) With your Arduino IDE open, write this program:

void setup() {

pinMode(LED_BUILTIN, OUTPUT); 		// Set built-in LED pin as output

}

void loop() {

digitalWrite(LED_BUILTIN, HIGH); 		// Turn on LED

delay(1000);                     			// Wait 1 second

digitalWrite(LED_BUILTIN, LOW);  		// Turn off LED

delay(1000);                     			// Wait 1 second

}

This program runs what programmers call an “infinite loop” or “forever loop,” meaning the program just runs over and over again until someone makes it stop.

In this program, the loop does this:

1. Turn on the built-in LED light.

2. Wait one second

3. Turn off the built-in LED light

4. Wait one second

Then it goes back to the beginning and does the loop all over again!

Pretty neat, huh?

Now that you’ve written the program, you need to download it to your microcontroller. Remember, like Mr. Fisher said, a microcontroller is like a miniature computer that focuses on one task. An Arduino-compatible microcontroller like the one we’re using here can only hold one program at a time! Very different than a powerful computer like your desktop or laptop or smart phone!

Here's how you download the program to your microcontroller.

Step 1: Connect your microcontroller to your computer with a USB cable, like this

(show sketch)

Step 2: Look for your device in the dropdown menu of the Arduino IDE

(show screenshot)

Step 3: Click the Upload button (show sketch)

That’s it! Now your program is uploaded to your microcontroller! And look what’s happening. The little LED on the controller has begun to blink!

Want to make your microcontroller blink a secret code like mine did when it was blinking a Dewey Decimal number? Head over to gavinholloway.com/programming and I’ll show you how to do it—number by number, blink by blink.

void setup() {

pinMode(LED_BUILTIN, OUTPUT);

}

void loop() {

blinkNumber(5);

blinkNumber(3);

blinkNumber(1);

delay(1000); // long pause between 1 and 9

blinkNumber(9);

blinkNumber(4);

blinkNumber(2);

delay(3000); // pause before repeating

}

void blinkNumber(int count) {

for (int i = 0; i < count; i++) {

digitalWrite(LED_BUILTIN, HIGH);

delay(200);

digitalWrite(LED_BUILTIN, LOW);

delay(200);

}

delay(600); // space between digits

}

Arduino Uno is a trademark of Arduino AG, a marvelous company who has open-sourced their designs to encourage the spread of STEM learning.