So now, with a simulated traffic light made, let’s look again at the page Marvin left for us in his book about making “The Beacon of Orinthgloth.”

(show sketch including instructions to make the blue light up for 2 seconds, then 0.5 seconds of nothing, then the white for 1.5 seconds, then 1 second of nothing, then the yellow for 1 second, then the green twice in a row for 0.75 seconds each with a 0.2 second delay, then the red for three seconds)

Now that you’ve programmed one of these, I bet you know how to do it too.

You can find a full step by step tutorial for The Beacon of Orinthgloth on my web site. Here’s the short version.

We built this circuit:

(show sketch)

And we wrote this code:

// Pin assignments (change to match your wiring)

const int BLUE_LED   = 2;

const int WHITE_LED  = 3;

const int YELLOW_LED = 4;

const int GREEN_LED  = 5;

const int RED_LED    = 6;

void setup() {

// Set LED pins as outputs

pinMode(BLUE_LED, OUTPUT);

pinMode(WHITE_LED, OUTPUT);

pinMode(YELLOW_LED, OUTPUT);

pinMode(GREEN_LED, OUTPUT);

pinMode(RED_LED, OUTPUT);

}

void loop() {

// 1. Blue LED on for 2 seconds

digitalWrite(BLUE_LED, HIGH);

delay(2000);

digitalWrite(BLUE_LED, LOW);

// 0.5 seconds of nothing

delay(500);

// 2. White LED on for 1.5 seconds

digitalWrite(WHITE_LED, HIGH);

delay(1500);

digitalWrite(WHITE_LED, LOW);

// 1 second of nothing

delay(1000);

// 3. Yellow LED on for 1 second

digitalWrite(YELLOW_LED, HIGH);

delay(1000);

digitalWrite(YELLOW_LED, LOW);

// 4. Green LED twice: 0.75s on, 0.2s off, 0.75s on

for (int i = 0; i < 2; i++) {

digitalWrite(GREEN_LED, HIGH);

delay(750);

digitalWrite(GREEN_LED, LOW);

if (i == 0) delay(200); // gap only between the two greens

}

// 5. Red LED on for 3 seconds

digitalWrite(RED_LED, HIGH);

delay(3000);

digitalWrite(RED_LED, LOW);

// Optional: short pause before repeating the sequence

delay(500);

}

Did you see that we did something different in this code? Those first lines that say const int BLUE_LED? We read about that in the Programming Microcontrollers book. I explain it in detail in the online tutorial.

So we finished it. And looked at it. And enjoyed our creation.

“Now what?” said Mateo.

Of course the only thing left for us to wonder about was that cryptic note at the bottom of the paper.

“Once finished, shine the Beacon atop Wickenshire Castle.”

(note to self….getting rid of the Don’t use the Internet portion of the story since I am frequently inviting users to come to my web site…the whole idea of bad guys chasing them isn’t working out in the story anyway)

“Now we have to shine the Beacon atop Wickenshire Castle,” I said.

“Where is that? Is it, like, in England?” said Mateo.

“Don’t bother looking it up,” said Charlotte. “I’ve already tried. There’s nothing to find.”

(note to self: Wickenshire Castle is a reference to a play structure inside Brimley Forge Children’s Museum)

Weirdly, I felt like I’d heard of Wickenshire Castle before.

Unable to figure out what Wickenshire Castle was, we soon lost interest in it.

(narrate ahead a few days)

Charlotte’s phone chimed. She pulled it from her pocket to look at it.

“Dad’s wondering why I’m not home yet,” she said. “I should go.”

“Me too,” sai

“Well, whatever it is, we’re not going to figure it out tonight,” said Charlotte. “I need to go